Automated guided vehicle system at Arburg

As if by magic

- Automated guided vehicle system connects two assembly halls

- Several autonomous transport vehicles for pallets and machine stands

- Integration into existing transport systems and traffic

Loßburg, 14.02.2024

The initial situation in Arburg's transport logistics was clear: the extended assembly areas created by a new hall made it necessary to expand the internal conveyor systems. The new, third assembly hall, which was built in 2019/2020 as an extension of the two existing halls, had to be connected logistically. The existing assembly halls are supplied from the central high-rack warehouse using an electric monorail system. The new transport system was designed to be driverless from the outset and integrates the new hall into the existing logistics chain, ensuring a continuous, smooth supply to all three halls.

Why was an automated guided vehicle system (AGV) chosen? Helmut Stumpp, Group Manager Transport, answers the question as follows: "First of all, the AGV offers much greater flexibility at pick-up and drop-off points than the existing automatic systems, such as the electric monorail system. Deliveries can be made directly to the worker and collections can be made directly at the 'workbench'. The entire system can be used in three shifts and can be expanded more quickly, cheaply and easily with additional transfers. This has enabled us to further increase our level of automation in assembly and we have managed without interfering with the building fabric of the halls, which was also very important."

The objective

It's easy to read: the AGV is to transport Arburg pallets with special dimensions, half-Euro and Euro pallets as well as machine bodies over a total travel distance of around 350 meters to the latest assembly hall. A total of seven automated guided vehicles (AGVs) with seven battery charging stations are used for this purpose - one for each vehicle. The terminals operate web-based on the basis of the existing Wi-Fi radio network at Arburg.

Six of the seven AGVs are automated forklift trucks of the "AMADEUS" type, which are used to transport pallets. The Arburg pallets with frames measure 1,400 x 1,000 x 350 millimetres and can carry a maximum weight of 1,750 kilograms. The Euro pallets with frame measure 1,200 x 800 x 330 millimeters and can be loaded with 1,000 kilograms. The size of the half-Euro pallets with frame is 600 x 800 x 300 millimetres and the load capacity is 750 kilograms.

There is also another AGV for machine stands with impressive dimensions of 5,000 x 1,700 x 2,250 millimetres, which in turn allows a payload of 1,800 kilograms. Among other things, the so-called "4-way AGV" can move forwards in "crab steering". This diagonal steering allows a vehicle to be moved crosswise so that the path of travel can be kept as narrow as possible. The vehicle is equipped with a plate conveyor and a data light barrier. An occupancy sensor is also fitted to check whether there is a load carrier on the slat conveyor.

With all AGVs, the goods to be transported do not protrude beyond the dimensions of the pallets or vehicles. It is also important to note that the automated guided vehicle system is only used indoors during assembly and is therefore not exposed to the weather.

Challenges in the project

An important aspect with a significant impact is the other "road users" on the planned AGV routes. At Arburg, this involves "normal" passenger traffic as well as tugger trains, bicycle traffic and various industrial trucks such as forklift trucks, low and high lift trucks. In order to be able to transport autonomously, the AGVs must therefore recognize obstacles and stop in front of them. To prevent damage, the flat forks of other industrial trucks in particular must be detected.

Prepared for everything

The navigation of the AGVs works with lasers in combination with environmental features. This achieves a positioning accuracy of +/- 20 millimeters. The vehicles drive to "their" charging station independently and in good time to recharge themselves.

For future expansion, routes of encapsulated layouts are also possible. The number of vehicles can be changed at any time during operation. This allows the company to react flexibly to future growth.

Transport and transfer points

The electric monorail system (EMS) and other industrial trucks transport all parts and components required for assembly from the high-rack warehouse to the three assembly halls. The AGV extension was planned and implemented in the passageway between the two existing assembly halls and the newest assembly hall, which, according to Helmut Stumpp, started transport in May 2023.

From June to August, the material flow was completed by a pallet lifter to the first floor of the new assembly hall. The transport volume of the extension is around 30 transports per hour, excluding machine body transports. These are only required to a lesser extent.

Example: Removal process from the machine column storage area

A total of 14 steps are required to remove a machine upright from the warehouse using the "4-way AGV" - a good example of the complexity of the processes required to operate an automated guided vehicle system smoothly.

First, the transport order for retrieval is created in the transport control system. The AGV then drives to the machine rack warehouse and registers in front of the access gate. The transport control system sets the retrieval order to "active" status if no other order is pending. The storage crane moves to the storage location (source), picks up the machine stand and clears the entry area for the AGV. The transport control system then gives the AGV the travel order to drive from its intermediate position into the machine column storage area. The warehouse signals entry clearance (gate clearance) to the AGV, the AGV drives in and signals "Position in warehouse reached" (release warehouse and "Close gate"). The "4-way AGV" is loaded, the warehouse reports the loading and the approach to the alternative location. The AGV now registers the exit, whereupon the warehouse issues the exit release. After the exit has taken place, the AGV reports in front of the gate again. The machine column storage system closes the gate and activates the storage crane again for new orders on the electric monorail system. Then comes the final step: the AGV moves the machine column to the lifter in the assembly hall and reports its arrival.

How the transport goods are handed over

The sources, i.e. the transfer points from the monorail overhead conveyor to the automated guided vehicle system, are divided into pallet delivery and machine body/machine column pick-up. The pallets are transferred from the overhead monorail to a chain conveyor, from which the AGV picks up the pallets on the opposite side. This avoids crossing traffic between the two transport systems.

The "4-way AGV" independently retrieves the machine bodies from the warehouse. The AGV and monorail overhead conveyor cross here. The lowering points of the AGV, which are the delivery points in the latest assembly hall, are located in the basement. Here, too, the delivery is split into two parts, again via a chain conveyor for the pallets.

The machine uprights are transferred via a second floor lifter with an attached slat conveyor and lifted to the first floor. On the first floor, both pallets and machine bodies are then transferred to the corresponding conveyor system.

System integration of the AGV

With the changeover to SAP EWM for the efficient management of warehouses and the processing and monitoring of goods movements in 2022, transport orders will be sent from SAP EWM to the transport control system via interfaces. From there, they are then distributed to the subordinate systems or interface systems. The AGV is connected to the transport control system. Communication in the interface is controlled via telegrams consisting of header and user data.

High satisfaction with AGVs at Arburg

The system has now been running in its entirety since January 2024. Is Arburg satisfied with the design and performance? Andreas Joos (Factory Planning & Processes) says: "As with every new system, there was a 'learning curve' at the beginning at Arburg, in which we familiarized ourselves with the technology and learned how the system works and reacts in everyday work.

The competent team from the conveyor technology group did a very good job in cooperation with the supplier DS Automotion. The line extension works and the target of the possible transport volume has been achieved.

Now it is time to use the knowledge gained here for further expansion in other areas. We are very satisfied with the progress of the project and look forward to what lies ahead."

Bilder

Das fahrerlose Transportsystem bei Arburg arbeitet mit verschieden großen Transportfahrzeugen. Die kleineren automatisierten Gabelstapler des Typs „Amadeus“ übernehmen den Palettentransport.

Das fahrerlose Transportfahrzeug (FTF) für Maschinenständer mit imposanten Ausmaßen von 5.000 x 1.700 x 2.250 Millimetern erlaubt eine Zuladung von 1.800 Kilogramm.

Die Paletten enden an einer Übergabestation mit Kettenförderer, an die die automatisierten Gabelstapler ihre Fracht übergeben.

Die Senken des FTS, also die Abgabepunkte in der neuesten Montagehalle, befinden sich im Untergeschoss (UG). Hier erfolgt die Abgabe zweigeteilt, und zwar einerseits über einen Kettenförderer für die Paletten, andererseits über einen Geschossheber mit aufgesetztem Plattenband.

Die FTF fahren ihre Ladestationen selbstständig an, um sich für den laufenden Betrieb funktionsfähig zu halten.

Die Wartepunkte – sogenannte Entscheidungspunkte – liegen als virtuelle Punkte auf dem Fahrweg und dienen den FTFs als vorgelagerten „Zwischenziele“, bis das endgültige Ziel bekannt bzw. frei ist.

Fotos: ARBURG

Foto Download:

Link

Pressemitteilung

Datei: input.doc

Zeichen: 8.741

Wörter: 1.191

Diese und weitere Pressemitteilungen finden Sie zum Download auch auf unserer Website unter www.arburg.com/de/presse/ (www.arburg.com/en/presse/)

Kontakt

ARBURG GmbH + Co KG

Pressestelle

Susanne Palm

Dr. Bettina Keck

Postfach 1109

72286 Loßburg

Tel.: +49 7446 33-3463

Tel.: +49 7446 33-3259

presse_service@arburg.com

Über Arburg

Das deutsche Familienunternehmen Arburg gehört weltweit zu den führenden Maschinenherstellern für die Kunststoffverarbeitung. Das Produktportfolio umfasst Allrounder-Spritzgießmaschinen mit Schließkräften zwischen 125 und 6.500 kN, Freeformer für die industrielle additive Fertigung sowie Robot-Systeme, kunden- und branchenspezifische Turnkey-Lösungen und weitere Peripherie.

In der Kunststoffbranche ist Arburg Vorreiter bei den Themen Energie- und Produktionseffizienz, Digitalisierung und Nachhaltigkeit. Das Programm „arburgXworld“ umfasst alle digitalen Produkte und Services und ist gleichzeitig der Name des Kundenportals. Die Strategien zur Ressourcen-Effizienz und Circular Economy sowie alle Aspekte und Aktivitäten dazu sind im Programm „arburgGREENworld“ zusammengefasst.

Zentrales Ziel von Arburg ist, dass die Kunden ihre Kunststoffprodukte vom Einzelteil bis zur Großserie in optimaler Qualität ressourcenschonend, nachhaltig und zu minimalen Stückkosten fertigen können. Zu den Zielgruppen zählen z. B. die Automobil- und Verpackungsindustrie, Kommunikations- und Unterhaltungselektronik, Medizintechnik und der Bereich Weißwaren.

Eine erstklassige Kundenbetreuung vor Ort garantiert das internationale Vertriebs- und Servicenetzwerk: Arburg hat eigene Organisationen in 26 Ländern an 36 Standorten und ist zusammen mit Handelspartnern in über 100 Ländern vertreten. Produziert wird in der deutschen Firmenzentrale in Loßburg. Von den insgesamt rund 3.800 Mitarbeitenden sind rund 3.200 in Deutschland beschäftigt und rund 600 in den weltweiten Arburg-Organisationen. Arburg ist zertifiziert nach ISO 9001 (Qualität), ISO 14001 (Umwelt), ISO 27001 (Informationssicherheit), ISO 29993 (Ausbildung) und ISO 50001 (Energie).

Weitere Informationen über Arburg finden Sie unter www.arburg.com